WaterHeaterStation - adcTest.py

from time import sleep

from max1238 import Max1238, InputMode

Vref = 4.096

def to_volts(raw: int) -> float:

return (raw / 4096.0) * Vref

adc = Max1238()

adc.setup_adc()

print("=== Single channel reads ===")

for i in range(12):

raw = adc.read_single(channel=i, mode=InputMode.SingleEnded)

print(f"CH{i}: {to_volts(raw):.3f} V")

print()

print("=== Read multiple (contiguous block) ===")

# Example: read channels 0..3 in one transaction

raws = adc.read_multiple(start_channel=0, count=4, mode=InputMode.SingleEnded)

for i, raw in enumerate(raws, start=0):

print(f"CH{i}: {to_volts(raw):.3f} V")

print()

print("=== Read range (dict) ===")

# Example: read channels 6..9 efficiently

vals = adc.read_range(6, 9, mode=InputMode.SingleEnded)

for ch, raw in vals.items():

print(f"CH{ch}: {to_volts(raw):.3f} V")

print(

Notes:

## GPIO

- GPIO 17 Valve control (Output)

- GPIO 21 Display A0

- GPIO 25 Display Reset

GPIO 18 and 26 are just used as mosi and cs for spi

## ADC (MAX1239KEEE+) using i2c

- CH 0  🔥 Temperature Transmitter Hot

- CH 1  🧊 Temperature Transmitter Cold

- CH 2  ⚙️ Flow Meter

- CH 3  ⏳ Future Use

- CH 4  🌡️ Ambient Temperature

- CH 5  ⏳ Future Use

- CH 6  ⏳ Future Use

- CH 7  ⏳ Future Use

- CH 8  ⏳ Future Use

- CH 9  ❌ Not Used

- CH 10 ❌ Not Used

- CH 11 ❌ Not Used

[max1239](https://www.analog.com/media/en/technical-documentation/data-sheets/max1236-max1239m.pdf)

ADC is using a i2c

## ADC config table

| BIT 7 | BIT 6 | BIT 5 | BIT 4 | BIT 3 | BIT 2 | BIT 1 | BIT 0 |

| --------------- | --------------- | --------------- | --------------- | --------------- |

| REG | SCAN1 | SCAN0 | CS3 | CS2 | CS1 | CS0 | SGL/^DIF |

for CH0 with single ended mode the config byte would be 0x1 00 0000 1

for CH1 with single ended mode the config byte would be 0x1 00 0001 1

for CH1 with single ended mode the config byte would be 0x1 00 0010 1