# Power Monitoring Software (ACS37800)

This section explains the Python code used to interface the ACS37800 power monitoring IC with the Raspberry Pi, perform calibration, and log electrical measurements.

## 1. System Configuration

This section defines the I²C communication parameters and register addresses used to communicate with the ACS37800.

- The device address is set to 0x60, which matches the hardware configuration.

- The Raspberry Pi uses I²C bus 1.

- Three main registers are used:

  - REG_RMS → contains VRMS and IRMS values

  - REG_PQ → contains active and imaginary power

  - REG_SPF → contains apparent power and power factor

These registers store data in 32-bit little-endian format and must be decoded properly.

## 2. File Management and Storage

The code defines a folder where all data and calibration files are stored.

- The calibration file (acs37800_cal.json) stores scaling factors and offsets.

- CSV files are generated automatically for logging measurements over time.

- The folder is created if it does not already exist.

This ensures all measurement data is saved consistently for later analysis.

## 3. Noise Filtering (Noise Floor Handling)

To avoid incorrect readings when inputs are floating or no load is present, noise thresholds are applied.

- Small raw values close to baseline are ignored.

- Voltage below 5 V is treated as 0 V.

- Current below 0.2 A is treated as 0 A.

This prevents “ghost readings” when the system is powered but no real signal is present.

## 4. Data Conversion Functions

Helper functions are used to convert raw register data into usable values.

- u16() converts unsigned 16-bit values.

- s16() converts signed 16-bit values.

- read32_le() reads 32-bit data from the sensor.

- pf_from_11bit() converts the power factor from its encoded format.

These functions ensure correct interpretation of sensor data.

## 5. Calibration Handling

The calibration system is critical for converting raw sensor data into real-world units.

### Calibration parameters:

- vrms_scale → converts raw voltage to volts

- irms_scale → converts raw current to amps

- vrms_offset → baseline voltage reading at 0 V

- irms_offset → baseline current reading at 0 A

The calibration values are saved in a JSON file and automatically loaded when the program starts.

## 6. Measurement Processing

The function get_values() reads sensor data and converts it into usable electrical values.

Steps:

- Read raw data from registers

- Extract voltage, current, and power values

- Apply offset correction

- Apply scaling factors

- Filter noise

- Compute estimated power using:

P=V×I×PF

If calibration is missing or communication fails, the function returns None.

## 7. Calibration Procedure

The code includes a guided calibration process.

### Steps:

- VRMS Offset

  - Turn off mains

  - Capture baseline voltage

- IRMS Offset

  - Turn on voltage, no load

  - Capture baseline current

- Voltage Scaling

  - Enter known line voltage (e.g., 240 V)

  - Compute voltage scale

- Current Scaling

  - Turn on heater (load)

  - Enter clamp meter reading

  - Compute current scale

The calibration values are then saved to a JSON file for future use.

## 8. Data Logging

The program continuously logs measurements into a CSV file.

Each row includes:

- Timestamp

- Vrms

- Irms

- Estimated Power

- Power Factor

- Raw sensor values

This allows post-processing and analysis using tools like Python or Excel.

## 9. Main Execution Loop

The main function controls the overall workflow:

- Load calibration file

- Ask user whether to recalibrate

- Continuously read sensor data

- Print results to terminal

- Save data to CSV file every second

The system runs indefinitely until manually stopped.

## 10. Overall System Function

This software enables:

- Real-time voltage and current monitoring

- Accurate power measurement using calibration

- Continuous data logging for analysis

- Robust operation with noise filtering

It is designed specifically for monitoring electric water heaters in a controlled lab environment.

# Main Code (Measurement and Logging)

This code is responsible for real-time communication with the ACS37800 sensor, processing the measured data, and logging the results for analysis.

#!/usr/bin/env python3

from smbus2 import SMBus

import time, json, os

from datetime import datetime

# -----------------------------

# User setup (your confirmed)

# -----------------------------

ADDR = 0x60

BUS  = 1

# 32-bit little-endian registers

REG_RMS   = 0x20   # [15:0]=VRMS(u16), [31:16]=IRMS(s16)

REG_PQ    = 0x21   # [15:0]=PACTIVE(s16), [31:16]=PIMAG(u16)

REG_SPF   = 0x22   # [15:0]=PAPP(u16), [26:16]=PF(11b signed), bit27 posangle, bit28 pospf

# Folder/file locations

FOLDER   = os.path.expanduser("./")

CAL_FILE = os.path.join(FOLDER, "acs37800_cal.json")

# Noise floors (tune if needed)

# These prevent "ghost" readings when VINP/inputs float (chip powered but mains/load removed)

NOISE_FLOOR_VRMS_CODES = 300    # raw codes near baseline treated as 0V

NOISE_FLOOR_IRMS_CODES = 80     # raw codes near baseline treated as 0A

NOISE_FLOOR_V_VOLTS    = 5.0    # Vrms below this -> show 0.0

NOISE_FLOOR_I_AMPS     = 0.20   # Irms below this -> show 0.0

# Power sign handling:

# If you want consumed power always positive, keep POWER_ABS=True

POWER_ABS = True

# -----------------------------

# Helpers

# -----------------------------

def u16(x): return x & 0xFFFF

def s16(x):

x &= 0xFFFF

return x - 0x10000 if (x & 0x8000) else x

def read32_le(bus, reg):

# Returns None on I2C error

try:

b = bus.read_i2c_block_data(ADDR, reg, 4)

except Exception:

return None

return b[0] | (b[1] << 8) | (b[2] << 16) | (b[3] << 24)

def pf_from_11bit(pf11):

# Signed 11-bit with 10 fractional bits (range about -1..+1)

pf11 &= 0x7FF

if pf11 & 0x400:

pf11 -= 0x800

return pf11 / (2**10)

def load_cal():

cal = {

# scales convert (raw - offset) -> engineering units

"vrms_scale": None,     # V per code

"irms_scale": None,     # A per code

# offsets are captured raw baselines

"vrms_offset": 0,       # VRMS_raw at 0V (mains/transformer off but chip powered)

"irms_offset": 0,       # IRMS_raw at 0A (voltage present but no load)

# saved metadata

"last_cal_time": None,

"line_vrms_used": None,

"clamp_irms_used": None

}

if os.path.exists(CAL_FILE):

try:

with open(CAL_FILE, "r") as f:

cal.update(json.load(f))

except Exception:

pass

return cal

def save_cal(cal):

os.makedirs(FOLDER, exist_ok=True)

with open(CAL_FILE, "w") as f:

json.dump(cal, f, indent=2)

def get_values(bus, cal):

r20 = read32_le(bus, REG_RMS)

r21 = read32_le(bus, REG_PQ)

r22 = read32_le(bus, REG_SPF)

if (r20 is None) or (r21 is None) or (r22 is None):

return None

vrms_raw = u16(r20)

irms_raw = s16(r20 >> 16)

pactive_raw = s16(u16(r21))

pimag_raw   = u16(r21 >> 16)

papparent_raw = u16(r22)

pf11 = (r22 >> 16) & 0x7FF

pf = pf_from_11bit(pf11)

# ---------- Raw-domain clamping (prevents floating-input ghosts) ----------

vr0 = int(cal.get("vrms_offset", 0))

ir0 = int(cal.get("irms_offset", 0))

if abs(vrms_raw - vr0) < NOISE_FLOOR_VRMS_CODES:

vrms_raw = vr0

if abs(irms_raw - ir0) < NOISE_FLOOR_IRMS_CODES:

irms_raw = ir0

# ------------------------------------------------------------------------

vrms = None

if cal["vrms_scale"] is not None:

vrms = (vrms_raw - vr0) * float(cal["vrms_scale"])

irms = None

if cal["irms_scale"] is not None:

irms = (irms_raw - ir0) * float(cal["irms_scale"])

# Engineering-domain clamping

if vrms is not None and vrms < NOISE_FLOOR_V_VOLTS:

vrms = 0.0

if irms is not None and abs(irms) < NOISE_FLOOR_I_AMPS:

irms = 0.0

# Power estimate using PF from chip

p_est = None

if (vrms is not None) and (irms is not None):

p_est = vrms * irms * pf

if POWER_ABS:

p_est = abs(p_est)

return {

"vrms_raw": vrms_raw,

"irms_raw": irms_raw,

"pactive_raw": pactive_raw,

"pimag_raw": pimag_raw,

"papparent_raw": papparent_raw,

"pf": pf,

"vrms": vrms,

"irms": irms,

"p_est": p_est,

}

def calibrate(bus, cal):

print("\n=== Calibration (Aligned to your setup) ===")

print("NOTE: Your chip is powered by separate supply.")

print("We will capture TWO baselines:")

print("  A) VRMS offset at 0V (mains/transformer OFF, chip still powered)")

print("  B) IRMS offset at 0A (voltage present, heater OFF / no load)")

print("Then we capture two scales (240V and clamp current).\n")

# A) Capture VRMS offset at 0V

print("A) VRMS offset (0V): Turn OFF mains/transformer so VINP should be ~0V.")

input("Press Enter to capture VRMS_raw offset (0V)...")

vals = get_values(bus, cal)

if vals is None:

print("I2C read failed. Check wiring/address.")

return

cal["vrms_offset"] = int(vals["vrms_raw"])

print(f"Captured VRMS offset = {cal['vrms_offset']} raw codes")

# B) Capture IRMS offset at 0A with voltage present

print("\nB) IRMS offset (0A): Turn ON mains (voltage present), but ensure NO LOAD (heater OFF).")

input("Press Enter to capture IRMS_raw offset (0A)...")

vals = get_values(bus, cal)

if vals is None:

print("I2C read failed. Check wiring/address.")

return

cal["irms_offset"] = int(vals["irms_raw"])

print(f"Captured IRMS offset = {cal['irms_offset']} raw codes")

# Voltage scale

print("\nC) Voltage scale: keep mains ON (normal operation).")

v_known = float(input("Enter TRUE line voltage Vrms (e.g., 240): ").strip())

input("Press Enter to capture VRMS_raw for scaling...")

vals = get_values(bus, cal)

if vals is None:

print("I2C read failed. Check wiring/address.")

return

vr = int(vals["vrms_raw"])

denom_v = float(vr - cal["vrms_offset"])

if abs(denom_v) < 10:

print("VRMS_raw - vrms_offset is too small. Is mains really ON? Is VINP driven?")

return

cal["vrms_scale"] = v_known / denom_v

cal["line_vrms_used"] = v_known

print(f"VRMS_raw={vr} => vrms_scale={cal['vrms_scale']:.10f} V/code")

# Current scale

print("\nD) Current scale: turn ON heater so current flows. Use clamp meter.")

i_known = float(input("Enter TRUE current Irms from clamp (e.g., 18.7): ").strip())

input("Press Enter to capture IRMS_raw under load...")

vals = get_values(bus, cal)

if vals is None:

print("I2C read failed. Check wiring/address.")

return

ir = int(vals["irms_raw"])

denom_i = float(ir - cal["irms_offset"])

if abs(denom_i) < 10:

print("IRMS_raw - irms_offset is too small. Increase load and try again.")

return

cal["irms_scale"] = i_known / denom_i

cal["clamp_irms_used"] = i_known

print(f"IRMS_raw={ir} => irms_scale={cal['irms_scale']:.10f} A/code")

cal["last_cal_time"] = datetime.now().isoformat(timespec="seconds")

save_cal(cal)

print(f"\nSaved calibration to: {CAL_FILE}\n")

def main():

os.makedirs(FOLDER, exist_ok=True)

cal = load_cal()

csv_name = datetime.now().strftime("acs37800_%Y%m%d_%H%M%S.csv")

csv_path = os.path.join(FOLDER, csv_name)

with SMBus(BUS) as bus, open(csv_path, "w") as f:

f.write("time,vrms,irms,p_est,pf,vrms_raw,irms_raw,pactive_raw,pimag_raw,papparent_raw\n")

print(f"Logging to: {csv_path}")

print("Commands:")

print("  c  -> full calibration (recommended)")

print("  Enter -> start using saved calibration\n")

cmd = input("> ").strip().lower()

if cmd == "c":

calibrate(bus, cal)

cal = load_cal()

while True:

t = datetime.now().isoformat(timespec="seconds")

vals = get_values(bus, cal)

if vals is None:

print(f"{t}  I2C READ ERROR")

time.sleep(1)

continue

vrms = vals["vrms"]

irms = vals["irms"]

p_est = vals["p_est"]

pf = vals["pf"]

vrms_s = "None" if vrms is None else f"{vrms:.2f}"

irms_s = "None" if irms is None else f"{irms:.2f}"

p_s    = "None" if p_est is None else f"{p_est:.1f}"

print(f"{t}  Vrms={vrms_s}  Irms={irms_s}  P={p_s} W  PF={pf:+.3f}  "

f"(raw vr={vals['vrms_raw']} ir={vals['irms_raw']})")

f.write(f"{t},{vrms if vrms is not None else ''},{irms if irms is not None else ''},"

f"{p_est if p_est is not None else ''},{pf},"

f"{vals['vrms_raw']},{vals['irms_raw']},{vals['pactive_raw']},"

f"{vals['pimag_raw']},{vals['papparent_raw']}\n")

f.flush()

time.sleep(1)

if __name__ == "__main__":

main()



# Calibration Code (Scaling and Offset Adjustment)

The calibration code is used to convert raw sensor readings into accurate real-world values. Without calibration, the sensor output cannot be trusted because it depends on the hardware setup, including transformers and current sensing.

{

"vrms_scale": 0.008774816277284195,

"irms_scale": 0.0016478218949951794,

"vrms_offset": 168,

"irms_offset": 16,

"last_cal_time": "2026-02-11T18:21:14",

"line_vrms_used": 240.0,

"clamp_irms_used": 18.8

}

